Rc plane task 3

Development Approach for a Competition RC Aircraft: Simulation-led vs Prototype-led﻿
1. Recommendation and assumptions﻿
I recommend a hybrid, simulation-first, prototype-verified approach: use quick, low-cost simulation to narrow the design, then build and test a physical prototype early to check what the models cannot predict.﻿
Assumptions:﻿
• Small team, four weeks, a tight budget, and one competition date that cannot move.﻿
• Simple, low-cost tools are available (XFLR5, OpenVSP, a CAD package, a kitchen scale, a thrust stand).﻿
• Foam, balsa or 3D-printed airframes can be built in days, so a physical prototype is cheap compared with a full-scale aircraft.﻿
2. Advantages and limitations﻿
Simulation-led design (analysis and models first, build later)﻿
• Advantages: cheap per iteration, tests many designs quickly, no risk of crashing hardware, gives insight into stability and performance trends.﻿
• Limitations: only as good as its assumptions; low-Reynolds-number flow, surface finish, build quality and weight growth are hard to model; setup takes time and skill; it can give false confidence.﻿
Prototype-led design (build and fly early)﻿
• Advantages: reveals real problems immediately (weight, balance, vibration, control response); needs little modelling skill; builds team hands-on knowledge.﻿
• Limitations: every iteration costs materials and build time; crashes destroy parts; it is hard to see why a change helped; few designs can be tried within four weeks.﻿
3. Comparison﻿
Criterion.﻿
Simulation-led.(S.l)         Prototype-led(P.l)﻿
Cost.        ﻿
  (S.l) Low per run.        ﻿
(P.l) Moderateperiteration(materials,repairs)﻿
Time.﻿
(S.l)Slow to set up,     fast to repeat     ﻿
(P.l)Fast to start  slow per cycle﻿
Accuracy﻿
(S.l)Good for trends, weaker for absolute values﻿
(P.l)Real result, but affected by build variation and weather﻿
Iteration speed﻿
(S.l)Very fast once the model exists﻿
(P.l)Limited by build and repair time﻿
Failure detection﻿
(S.l)Finds design-level issues (stability, sizing); misses build defects﻿
(P.l)Finds real-world issues (weight, balance, structure, controls)﻿
Final performance﻿
(S.l)High if the model is validated﻿
(P.l)High if enough iterations fit in the schedule﻿
Neither approach alone is best. Simulation is cheap but can be wrong; prototyping is honest but slow. Under a hard four-week limit, the best use of both is to simulate first and prototype early.﻿
4. Proposed workflow﻿
1. Mission requirements: read the competition rules (payload, size, weight, takeoff distance, flight tasks) and define targets.﻿
2. Hand calculations: estimate weight, wing area, wing loading, stall speed, required thrust and battery size.﻿
3. Simulation: model the airframe in XFLR5 or OpenVSP and compare configurations.﻿
4. Design and CAD: finalise geometry and mass distribution.﻿
5. Manufacturing: build a first prototype quickly with cheap materials.﻿
6. Ground and flight testing: weigh, balance, thrust-test, glide-test, then fly.﻿
7. Refinement: compare results with predictions, correct the model, and rebuild.﻿
Figure 1 (insert): development flowchart of the seven steps above, with loops from testing back to simulation and design.﻿
5. Parameters: simulate versus verify﻿
Study in simulation:﻿
• Wing area, aspect ratio and airfoil, to size lift and drag (XFLR5).﻿
• Centre of gravity and static margin, to check pitch stability.﻿
• Lift-to-drag ratio and drag polar, to estimate power and endurance.﻿
• Tail volume and control authority, to predict stability and response.﻿
Verify physically:﻿
• Actual weight and centre-of-gravity position.﻿
• Static thrust and battery performance under load.﻿
• Control surface direction, throws and stiffness.﻿
• Structural strength of wing, joints and landing gear.﻿
• Takeoff distance and glide ratio.﻿
6. Validating simulations with measurements﻿
Treat the model as a hypothesis. Measure real values, feed them back, and compare predictions with results:﻿
• Weigh the aircraft and enter the real mass and CG into the model.﻿
• Measure static thrust and compare it with the propulsion estimate.﻿
• Do a glide test and compare the measured glide ratio with the predicted lift-to-drag ratio.﻿
• Fly with a simple data logger or video and compare stall speed and takeoff run with predictions.﻿
• Accept the model when predictions are within an agreed tolerance (for example 10 to 15%); otherwise correct the inputs, such as drag from surface roughness or extra weight.﻿
7. Case: simulated plane flies, prototype cannot take off﻿
Plausible causes:﻿
• Prototype heavier than the model (glue, wiring, battery, paint).﻿
• Centre of gravity wrong, causing nose-heavy or tail-heavy behavior.﻿
• Thrust lower than predicted (wrong propeller, battery voltage sag, motor or ESC mismatch).﻿
• Wing incidence or twist differs from the model; warped wing or rough surface adds drag.﻿
• Control surfaces reversed, binding or with too little throw.﻿
• Runway conditions, wind or too short a takeoff distance.﻿
Diagnostic plan, from cheapest check to most expensive, changing one variable at a time:﻿
1. Weigh the aircraft and check CG against the design value.﻿
2. Check control direction and throws on the ground.﻿
3. Run a static thrust test with a fully charged battery under load.﻿
4. Inspect wing incidence, warps and structural damage.﻿
5. Hand-launch glide test to separate aerodynamic problems from propulsion problems.﻿
6. Update the simulation with the measured weight, thrust and CG and re-run it; if the model now agrees, fix the cause it identifies.﻿
8. Tools﻿
XFLR5 analyses low-Reynolds-number airfoils and wings, which suits RC aircraft. OpenVSP builds parametric aircraft geometry for quick comparison. Fusion 360 or SolidWorks gives CAD and mass properties. ANSYS offers detailed CFD and structural analysis but is slow and probably too heavy for a four-week project. Running these tools is not mandatory; choosing them with reasons is.﻿
9. Bonus: four-week schedule﻿
Week﻿
Activities﻿
Contingency﻿
1﻿
Read rules, set requirements, hand calculations, XFLR5 or OpenVSP comparison﻿
Half a day spare﻿
2﻿
CAD, build prototype 1, ground tests (weight, CG, thrust)﻿
One day spare for failed parts﻿
3﻿
Test flights, compare data with model, fix and rebuild﻿
Spare airframe materials, weather days﻿
4﻿
Final build, practice flights, competition-ready checks﻿
Reserved last 3 days, no new changes﻿
10. Conclusion﻿
A hybrid approach gives the best result for a small team: simulate first to avoid obvious mistakes, then build early and measure everything so the model can be corrected. This keeps cost and risk low while giving enough physical iterations to be competition-ready.﻿
References﻿
1. XFLR5 project. http://www.xflr5.tech﻿
2. OpenVSP. https://openvsp.org﻿
3. Airfoil Tools. http://airfoiltools.com﻿
4. Raymer, D. P., Aircraft Design: A Conceptual Approach, AIAA.﻿
5. Anderson, J. D., Fundamentals of Aerodynamics, McGraw-Hill.